# IPL Wavelength Protocol & Treatment Guide

This guide outlines common IPL wavelengths, target chromophores, recommended indications, and treatment parameters for aesthetic providers using IPL devices such as Zemits. Always refer to your specific device’s user manual and training before treatment.

| Wavelength (nm) | Primary Target | Indications | Skin Type | Fluence (J/cm²) | Pulse Duration (ms) |
| --- | --- | --- | --- | --- | --- |
| 420 nm | Porphyrins (acne bacteria) | Acne, active pustules | I–III | 10–15 | 20–50 |
| 530 nm | Melanin, superficial hemoglobin | Sun spots, freckles, redness, facial veins | I–III | 12–20 | 20–60 |
| 560 nm | Oxyhemoglobin | Rosacea, facial telangiectasia, light vascular lesions | I–IV | 12–22 | 20–50 |
| 590 nm | Hemoglobin & pigment | Photorejuvenation, pigment, vessels | I–IV | 14–24 | 20–50 |
| 640 nm | Deeper melanin & hemoglobin | Hair removal (light to medium skin), vascular lesions | I–IV | 18–30 | 20–50 |
| 690–700 nm | Melanin (deep) | Hair removal, coarse dark hair | I–IV | 20–35 | 30–60 |
| 755 nm | Melanin (deepest) | Hair removal (type I–III), coarse hair | I–III | 22–38 | 30–60 |

⚠️ Note: Always perform a test spot and adjust based on client tolerance and response. Use caution on skin types IV–VI. Avoid sun exposure before and after treatments.