# Zemits DuoCratus IPL & Diode Laser Protocol Guide

Hello Gorgeous Med Spa – Treatment Manual

## 📌 Device Overview

The Zemits DuoCratus combines an 808 nm Diode Laser for effective hair removal with a multi-filter IPL system for treating pigmentation, acne, vascular concerns, and photoaging. This guide outlines each filter's clinical use, target chromophore, and suggested treatment settings.

## 🎯 IPL Filter Protocols

| Filter (nm) | Primary Targets | Indications | Suggested Fluence (J/cm²) |
| --- | --- | --- | --- |
| 480 nm | Porphyrins | Acne, active pustules | 10–15 |
| 530 nm | Melanin, Hemoglobin | Pigmentation, redness, facial veins | 12–20 |
| 590 nm | Oxyhemoglobin | Rosacea, telangiectasia, deeper pigment | 14–24 |
| 640 nm | Deeper melanin | Hair removal (light skin), vascular | 18–28 |
| 690 nm | Deep melanin | Coarse/dense hair removal | 20–35 |

## 🧊 808 nm Diode Laser Protocol

• Use for long-term hair removal on skin types I–V

• Energy: 10–60 J/cm² depending on Fitzpatrick type

• Pulse width: 5–400 ms | Frequency: 1–10 Hz

• Spot Size: 13 × 13 mm | Cooling: Sapphire contact tip

## 🤖 Smart Mode vs. Advanced Mode

• Smart Mode: AI-based adjustment of parameters based on real-time skin scan

• Advanced Mode: Manual selection of filter, fluence, pulse duration

## 📋 Treatment Protocol Flow

1. Conduct client consultation & AI skin analysis.

2. Select treatment mode and appropriate IPL filter or diode.

3. Perform test spot with low fluence to assess skin reaction.

4. Adjust parameters based on tolerance and desired depth.

5. Use cooling gel and provide post-care: SPF, no sun 48 hrs.

⚠️ Always follow clinical guidelines and device training protocols.